May 15, 2009

Instructions for Switching Beam-Modes

(High-Energy destinations only)

OLIS => High Energy

- Insert source Faraday cups (IOS:FC3/6, ITW:FC5, ITE:FC7, and IMS:FC3)

- Insert SEBT:BB20.

- Set the Target Ion Source (ITE/W) beam mode to one of the “Target LE Modes”.

- Set the IOS Beam mode to the desired HE / ISAC-II destination.

- Close IOS:B13 and IOS:B10 (requires IOS:AC3C off for movement).

- Turn on AC switch IOS:AC3C and the optics from IOS:B10 to IOS:B13.

- Load the appropriate setpoints for IOS:Q9 and ILT:Q33 (theoretical or "established tune" values).

- Request the safety enable on the source Faraday cups.

ITE/W => High Energy

- Insert source Faraday cups (ITW:FC5, ITE:FC7, IMS:FC3, and IOS:FC3/6).

- Insert SEBT:BB20.

- Set the IOS Beam mode to one of the “OLIS LE Modes”.

- Set the Target Ion Source (ITE/W) beam mode to the desired HE / ISAC-II destination.

- Turn off AC switches IOS:AC3C and ILE:AC2C.

- Open IOS:B13* and ILE:B1.

- Load the appropriate setpoints for ILT:Q26 and ILT:Q33 (theoretical or "established tune" values).

- Request the safety enable on the source Faraday cups.

*Note:  Opening IOS:B13 w/out tripping the safety system requires:

for ISAC-I:  DSB:MB0 OFF AND no magnetic field readback

for TUDA (or anywhere while DRAGON cage open):  HEBT3:MB0 AND HEBT3:MB1 selected (as opposed to HEBT2)