ISAC-II SC cavity manual start from EPICS interface

V. Zvyagintsev

30/07/2013

ISAC-II SC Cavity manul start from EPICS interface (when auto-on fails);

‘Vladimir’s Method,’ 2013:

- RF CONTROL

  - Leave amplitude (‘Voltage’) at setpoint.

  - Open Loop Drive ~500.

  - CW mode.

  - RF on.

  - Check amplitude readback:  it should be at least several hundreds.

    - If there is no amplitude readback, turn off cavity and turn on again (RF on) until amplitude readback attained.

    - If i. doesn’t work, switch cavity to pulse regime, wait ~5 min. Switch cavity back to CW looking for amplitude readback.  Make several attempts if necessary.

- COUPLER LOOP

  - Check Forward Power (Pfwd):  it should be less than 200W.

    - If Pfwd exceeds 200W, reduce Open Loop Drive to reduce Pfwd below 200W.

- RF CONTROL

  - Lock amplitude.

  - If amplitude readback matches setpoint, ‘AUTO-ON.’

  - Once ‘Cavity Ok’ green, procedure successful.

Figure 1 – EPICS SCRF cavity device page.