TRIUMF EMERGENCY  FLOOR WARDENS

Fire Prevention Duties

TRIUMF Emergency Floor Wardens will check their assigned areas for:

- Accumulation of combustible material, rubbish or flammable liquids in excess of allowable quantities.

- Dangerous ignition sources, i.e. worn extension cords, oily rags, covered baseboard heaters, overheating equipment.

- Exit lights not operating or not in good order and poor lighting in the halls and stairwells.  NOTE: All lighting problems must be reported to Plant Group.

- Faulty fire and exit doors, i.e. self-closing hardware or mechanism in poor shape. NOTE: Any approved fire door be it manual or controlled that is discovered held open by a wedge or weighted device must be closed upon exiting your area during a fire alarm.

- Exit routes obstructed (this means of egress that leads from a floor area it serves, including any doorway leading directly from a floor area it serves, to a thoroughfare or an approved open space).

- Portable fire extinguishers obstructed, not in good order or otherwise not ready to use.

- IMPORTANT: Report any dangerous /  hazardous process that may be scheduled at the same time a scheduled fire drill is to take place.

Any of the above, or any other fire hazards that are discovered, must be reported immediately to the Fire Safety Director (Gord Wood…7495), Deputy Fire Safety Director (Gord Roy…7328) or 500 MeV Operations 7333

# Upon Hearing a Fire Alarm

- Make sure all personnel are evacuating as you do a sweep of your area.  You may assist personnel with evacuation if available, i.e. shutting down a hazardous process or assisting physically handicapped personnel.

- All personnel are to evacuate to ground level from the buildings via the nearest fire exit…DO NOT USE THE ELEVATORS!!!

- Being the last to evacuate, make sure any fire doors with wedges or weighted devices holding them open are removed.  Never go in the opposite direction of safe egress, i.e. down or up stairs that lead away from ground level, into rooms with no other safe exit.

- Once you have safely exited your area stay a safe distance from your building, but near enough to help inform fire fighting personnel of the status of your building.  Identify yourself as a TRIUMF Emergency Floor Warden.

- When an “ALL CLEAR” has been given by the Vancouver Fire & Rescue Services and announced over the PA system or by an Emergency Floor Warden handling ‘COMMUNICATIONS’ …you may allow personnel to return to their building.

IMPORTANT: Make sure to assign a designate to your area when you are away on business, holidays or due to illness.